Multi-Router Network Lab

Static Routing, RIP, and OSPF on a Shared Topology — Packet Tracer

# 1. Objective

Build one physical network topology in Cisco Packet Tracer, then demonstrate three different routing methods — static routes, RIP, and OSPF — on identical copies of that topology. The exercise also includes VLAN segmentation, subnetting practice, and baseline security hardening on all network devices.

# 2. Topology Design

The lab uses 6 routers in a partially-meshed core, 2 access switches, and 2 end devices (a server and a PC), matching the whiteboard diagram provided by the instructor.

Router roles:

- R1, R2 — top-tier core routers, directly connected to each other

- R3 — left edge router, connects down to Switch1 (Server side)

- R6 — right edge router, connects down to Switch2 (PC side)

- R4, R5 — mid-tier routers providing redundant paths between the two sides

Each router connects to its neighbors using GigabitEthernet interfaces and Copper Straight-Through cables. In total there are 8 router-to-router links, forming a mesh with redundant paths rather than a single chain.

# 3. IP Addressing Plan

3.1 Point-to-Point Links (10.0.0.0/24)

The instructor specified 10.0.0.0/24 for all router-to-router (P2P) links. This block was broken into 8 individual /30 subnets — each /30 provides exactly 2 usable addresses, which is all a point-to-point link needs (one address per router).

| Link | Subnet | Network Addr | Usable Range | Broadcast | Assigned IPs |
| --- | --- | --- | --- | --- | --- |
| R1 – R2 | 10.0.0.0/30 | 10.0.0.0 | .1 – .2 | 10.0.0.3 | R1=.1, R2=.2 |
| R1 – R3 | 10.0.0.4/30 | 10.0.0.4 | .5 – .6 | 10.0.0.7 | R1=.5, R3=.6 |
| R1 – R4 | 10.0.0.8/30 | 10.0.0.8 | .9 – .10 | 10.0.0.11 | R1=.9, R4=.10 |
| R2 – R5 | 10.0.0.12/30 | 10.0.0.12 | .13 – .14 | 10.0.0.15 | R2=.13, R5=.14 |
| R2 – R6 | 10.0.0.16/30 | 10.0.0.16 | .17 – .18 | 10.0.0.19 | R2=.17, R6=.18 |
| R4 – R5 | 10.0.0.20/30 | 10.0.0.20 | .21 – .22 | 10.0.0.23 | R4=.21, R5=.22 |
| R3 – R4 | 10.0.0.24/30 | 10.0.0.24 | .25 – .26 | 10.0.0.27 | R3=.25, R4=.26 |
| R5 – R6 | 10.0.0.28/30 | 10.0.0.28 | .29 – .30 | 10.0.0.31 | R5=.29, R6=.30 |

Each subnet block is 4 addresses wide, so the network address always jumps by 4 (.0, .4, .8, .12 …). The two usable addresses always sit in the middle of the block; the first and last addresses in each block are reserved as the network and broadcast address and cannot be assigned to a device.

3.2 LAN / Access-Side Subnets

| Side | VLAN | Subnet | Gateway (Router) | End Device |
| --- | --- | --- | --- | --- |
| Left (Server) | VLAN 3010 | 172.16.0.0/28 | R3 = 172.16.0.1 | Server = 172.16.0.2 |
| Right (PC) | VLAN 4010 | 192.168.0.0/28 | R6 = 192.168.0.1 | PC = 192.168.0.2 |

# 4. Physical Setup and Interface Configuration

4.1 Cabling

- Placed 6 routers (Cisco 1941 or 2911) and cabled all 8 core links using Copper Straight-Through cables into GigabitEthernet ports.

- Placed 2 switches (2960), connected R3 → Switch1 and R6 → Switch2.

- Connected the Server to Switch1 and the PC to Switch2.

4.2 Configuring a Router Interface (example: R1–R2 link)

The same 5-step pattern was repeated for every one of the 8 links, only the IP addresses and interface numbers changed.

! On R1
enable
configure terminal
interface GigabitEthernet0/0
ip address 10.0.0.1 255.255.255.252
no shutdown
exit

! On R2
enable
configure terminal
interface GigabitEthernet0/0
ip address 10.0.0.2 255.255.255.252
no shutdown
exit

Verification on either router:

show ip interface brief
ping 10.0.0.2

Note: the very first ping across a newly configured link commonly shows one dropped packet (e.g. 80% success). This is normal — it is caused by the ARP resolution delay (the router must first discover the neighbor's MAC address before the actual ICMP packet can be sent). Running the same ping a second time typically returns 100% success once the MAC address is cached.

# 5. Switch and VLAN Configuration

VLANs 3010 and 4010 fall inside Cisco's extended VLAN range (1006–4094). By default, a switch in VTP server mode blocks the creation of extended VLANs. The fix was to set the switch to VTP transparent mode, which removes that restriction and is appropriate for a small, standalone lab topology.

enable
configure terminal
vtp mode transparent
vlan 3010
name LAN_Server
exit
interface FastEthernet0/1
switchport mode access
switchport access vlan 3010
exit
interface FastEthernet0/2
switchport mode access
switchport access vlan 3010
exit

The same pattern was repeated on Switch2 with VLAN 4010. On the router side, R3 and R6's interfaces facing their switches were addressed directly (no sub-interfaces needed, since each link only carries a single VLAN):

! R3
interface GigabitEthernet0/2
ip address 172.16.0.1 255.255.255.240
no shutdown

End devices (Server, PC) were given static IPs, subnet masks, and default gateways through their Desktop → IP Configuration screens.

# 6. Cloning the Topology for Three Routing Methods

Because the assignment required demonstrating static routes, RIP, and OSPF on the same physical design, the topology was built and fully IP-addressed once — with zero routing configured — and then branched into three separate files before any routing was added.

Checkpoint before cloning:

- All 8 router-to-router links show Status: up / Protocol: up with correct IPs

- Both VLAN gateway interfaces (R3, R6) are configured and up

- Server and PC have correct IP / mask / gateway and can ping their local router

- No ip route, router rip, or router ospf commands exist anywhere yet

Cloning steps in Packet Tracer:

- File → Save As → Topology-Static.pkt

- File → Save As → Topology-RIP.pkt

- File → Save As → Topology-OSPF.pkt

This produced three identical files sharing the same cabling and interface addressing. Each file was then opened independently, and only the relevant routing method was configured inside it — keeping the three methods completely isolated from one another for clean comparison and grading.

# 7. Static Routing (Topology-Static.pkt)

Rather than configuring every possible redundant path, one working end-to-end path was chosen: Server → R3 → R1 → R2 → R6 → PC. Static routes were added only on the 4 routers along this path (R3, R1, R2, R6); R4 and R5 were left out of this file since they are not part of the chosen path.

! R3 - route to PC subnet, via R1
ip route 192.168.0.0 255.255.255.240 10.0.0.5

      
! R1 - routes to both LAN subnets
ip route 172.16.0.0 255.255.255.240 10.0.0.6
ip route 192.168.0.0 255.255.255.240 10.0.0.2

      
! R2 - routes to both LAN subnets
ip route 172.16.0.0 255.255.255.240 10.0.0.1
ip route 192.168.0.0 255.255.255.240 10.0.0.18

      
! R6 - route to Server subnet, via R2
ip route 172.16.0.0 255.255.255.240 10.0.0.17

Each ip route command follows the pattern: destination network, its subnet mask, and the IP of the next-hop neighbor along the chosen path. Verified with show ip route (routes appear with an S code) and confirmed with ping 192.168.0.2 from the Server and ping 172.16.0.2 from the PC — both succeeded once ARP had resolved across all 5 hops.

# 8. RIP v2 (Topology-RIP.pkt)

Unlike static routing, RIP requires every router in the topology to participate — each router advertises its own directly connected networks, and RIP dynamically calculates paths for all 6 routers, including the redundant ones (R4, R5) that were skipped in the static file.

enable
configure terminal
router rip
version 2
network <connected network 1>
network <connected network 2>
network <connected network 3>
no auto-summary
exit

Example — R3 (connects to R1, R4, and the Server LAN):

router rip
version 2
network 10.0.0.4
network 10.0.0.24
network 172.16.0.0
no auto-summary
exit

version 2 was required because RIPv1 does not support classless (variable-length) subnet masks like the /30s and /28 used in this design. no auto-summary prevents RIP from collapsing the subnets back to their classful boundaries, which would break routing between them.

After all 6 routers were configured, show ip route showed routes marked with an R code for every subnet, including some with two equal-cost paths (a direct result of the mesh topology's redundancy). End-to-end ping between Server and PC succeeded in both directions, and tracert confirmed traffic flowing across the dynamically learned path.

# 9. OSPF (Topology-OSPF.pkt)

OSPF was configured on all 6 routers using a single backbone area (area 0), appropriate for a topology this size. OSPF network statements use a wildcard mask, which is the inverse of a normal subnet mask.

| Subnet Type | Subnet Mask | Wildcard Mask |
| --- | --- | --- |
| /30 (P2P links) | 255.255.255.252 | 0.0.0.3 |
| /28 (LAN subnets) | 255.255.255.240 | 0.0.0.15 |

Example — R6 (connects to R2, R5, and the PC LAN):

router ospf 1
network 10.0.0.16 0.0.0.3 area 0
network 10.0.0.28 0.0.0.3 area 0
network 192.168.0.0 0.0.0.15 area 0
exit

As each router was configured, %OSPF-5-ADJCHG log messages confirmed neighbor adjacencies forming and reaching the FULL state. Once all 6 routers were done and had time to converge, show ip route displayed O routes to every subnet in the topology — including 172.16.0.0/28 and 192.168.0.0/28 — with some subnets showing multiple equal-cost paths thanks to the mesh design. End-to-end connectivity was confirmed with successful pings between the Server and PC in both directions.

# 10. Security Hardening

After all three routing files were completed and verified, baseline security settings were applied identically to all 8 devices (6 routers + 2 switches):

enable
configure terminal
enable secret <password>
line console 0
password <password>
login
exit
line vty 0 4
password <password>
login
exit
service password-encryption
end
write memory

- enable secret — sets an encrypted privileged-EXEC password (stronger than the older enable password)

- line console 0 / line vty 0 4 — require a password for local console access and remote (Telnet) access

- service password-encryption — obscures passwords in show running-config so they aren't stored in plain text

- write memory — saves the running configuration to startup-config so settings persist across reloads

# 11. Summary

The same 6-router, 2-switch physical topology was used to demonstrate three distinct routing approaches, each verified independently:

| Method | How routes were built | Result |
| --- | --- | --- |
| Static | Manually entered ip route commands on 4 routers along one chosen path | Server ↔ PC reachable |
| RIP v2 | Dynamically learned via network statements on all 6 routers | Server ↔ PC reachable, redundant paths available |
| OSPF (Area 0) | Dynamically learned via wildcard-mask network statements, area 0, on all 6 routers | Server ↔ PC reachable, redundant equal-cost paths |

All three configurations were saved as separate .pkt files (Topology-Static.pkt, Topology-RIP.pkt, Topology-OSPF.pkt) sharing identical cabling and interface addressing, then hardened with baseline password and encryption settings to meet the instructor's security-standards requirement.